# Motors Selected:

## Motor Specifications

- Power: 450 W

- Voltage: 48 V

- Torque: 1.43 Nm

- Speed: 3000 RPM

- Gearbox: Planetary

- Gear Ratio: 16:1

https://peacosupport.com/brushless-dc-motors/high-torque-brushless-dc-motor/450w-brushless-dc-motor

# Motor Power Requirements:

## 1. Required Wheel Speed

Given:
 v = 11 mph
 r = 12 in = 0.3048 m

Velocity relationship:
 v = 2.236936 * (RPM_out * 2 * pi * r) / 60

Solve for RPM_out:
 RPM_out = (v * 60) / (2.236936 * 2 * pi * r) (due to us using mph the 2.236936 is the number needed to convert from m/s)

Substitute values:
 RPM_out = (11 * 60) / (2.236936 * 2 * pi * 0.3048)

RPM_out ≈ 155 RPM

## 2. Motor RPM

Motor_RPM = RPM_out * Gear_Ratio

Motor_RPM = 155 * 16

Motor_RPM ≈ 2480 RPM

## 3. Angular Velocity

omega = (2 * pi / 60) * Motor_RPM

omega = (2 * pi / 60) * 2480

omega ≈ 259.7 rad/s

## 4. Mechanical Power (Average Load)

tau_avg = 0.83 Nm

P_mech = omega * tau_avg

P_mech = 259.7 * 0.83

P_mech ≈ 215.6 W

## 5. Electrical Power

efficiency = 0.75

P_elec = P_mech / efficiency

P_elec = 215.6 / 0.75

P_elec ≈ 287.5 W

## 6. Current per Motor

I = P_elec / V

I = 287.5 / 48

I ≈ 6.0 A

# Battery Power Requirements:

## 7. Total Power (Two Motors)

P_total = 2 * P_elec

P_total = 2 * 287.5

P_total ≈ 575 W

## 8. Total Current

I_total = P_total / V

I_total = 575 / 48

I_total ≈ 12.0 A

## 9. Battery Capacity

Depth_of_Discharge = 0.80
 t = 2(hours)

Ah = (t * I_total) / 0.80

Ah = (t * 12.0) / 0.80

Ah = 30Ah