# Purchasable batteries (≤ 48 V) sorted by weight and comparison with user‑provided listing

Below is a comparison of the lithium battery options previously identified (only those at ≤ 48 V were kept) with the larger RELiON/AGM batteries listed in the user’s uploaded table. Each battery meets the user’s 230.65 Wh usable‑energy and ≈2.02 kW peak‑power requirement. I focus on weight (lightest to heaviest) and price, then highlight differences versus the user’s table.

| Rank (lightest→heaviest) | Battery & chemistry (≤ 48 V) | Key specifications | Weight | Approx. price* | Warranty & cycle life | Comparison with user’s table |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | FLEX 24 V 12 Ah Lithium‑ion battery (FX0231‑1) | 24 V, 12 Ah (≈288 Wh) pack designed for the FLEX tool platform. The spec sheet lists power 2,520 W and the battery weighs 4 lb (1.8 kg)[1]. The manufacturer notes that registering within 30 days provides a 5‑year limited warranty (3 years if unregistered)[2]. | ≈4 lb (1.8 kg)[1] | ≈US $280–340 (Global Industrial lists US $279.99, Canadian dealers about CA $339.99). Shipping to Boise for a 4‑lb item is usually under US $15 or free when the retailer offers free shipping on orders >US $149. | 5‑yr limited warranty with product registration[2]; cycle life not specified (tool batteries are typically rated for hundreds of cycles). | The user’s table lists RELiON RB24V52 at 29.1 lb and US $929.95. The FLEX battery is far lighter and cheaper but offers lower usable energy (288 Wh vs 1,280 Wh) and a shorter warranty. It still meets the 2.02‑kW requirement (≈105 A peak at 24 V). |
| 2 | Vatrer 36 V 50 Ah LiFePO₄ trolling‑motor battery | A 38.4 V (36 V nominal) LiFePO₄ pack with 1920 Wh energy and a BMS rated for 50 A continuous / 100 A peak (10 s)[3]. The specification table states that the battery weighs 22 lb and has an IP67 ABS enclosure[4]. Warranty is 5 years[4]. | ≈22 lb (10 kg)[4] | US $359.99[5]. Many marine-battery retailers offer free ground shipping in the continental US; shipping to Boise for a 22‑lb item is typically around US $20–30 if not free. | 5‑yr warranty[4]; the manufacturer claims 6,000 + cycles at 80 % depth of discharge[6]. | The user’s table lists RELiON RB36V40 at 30.9 lb and US $1,089.95. The Vatrer battery is lighter (22 lb vs 30.9 lb) and much cheaper while providing greater capacity (50 Ah vs 40 Ah) and similar peak power. It has a shorter warranty (5 yr vs 10 yr), but the cycle‑life claim (6,000+ cycles) exceeds RELiON’s 3,500‑cycle rating. |
| 3 | RELiON InSight 48 V‑LT 30 Ah LiFePO₄ battery (cold‑weather version) | A 51.2 V battery designed for golf carts and utility vehicles. The product page states a recommended continuous discharge of 45 A, 100 A max continuous, and 300 A peak for 10 s[7]. It delivers 1.536 kWh of energy and weighs 34.4 lb (15.6 kg)[8][9]. RELiON notes a 10‑year warranty and cycle life of 3,500+ cycles at 80 % DoD[10]. | ≈34.4 lb (15.6 kg)[9] | US $1,249.95[11]. Ground shipping to Boise (a 34‑lb package) typically costs US $30–40, but many battery vendors include free shipping at this price point. | 10‑yr warranty with 3,500+ cycle lifespan[10]; self‑heating for cold weather and smart BMS with CANbus[12]. | In the user’s table, the RELiON RB48V100 weighs 136.7 lb and costs US $3,495.95. The InSight 48 V‑LT pack is dramatically lighter (34.4 lb vs 136.7 lb) and cheaper while still meeting the peak‑power requirement (100 A continuous ≈5 kW at 51.2 V). Its capacity (30 Ah) is lower than the RB48V100’s 100 Ah, but sufficient to satisfy the user’s minimum energy needs. |
| 4 | Impulse Lithium 36 V 100 Ah “Platinum Series” battery | A 36 V LiFePO₄ battery targeting trolling‑motor and marine applications. The product page lists 60 A continuous discharge, 100 A peak discharge, 100 Ah capacity and 60 lb weight[13]. Impulse Lithium offers a ten‑year warranty on lithium batteries[14]. | ≈60 lb (27 kg)[13] | US $2,199.00[15]. Free shipping is often offered on orders from Impulse Lithium; otherwise, shipping a 60‑lb battery to Boise might cost ~US $50–70. | 10‑yr warranty[14]; warranty terms require product registration within 30 days and guarantee the battery will be free from defects for ten years. | The user’s table does not include a 100 Ah 36‑V option, so this battery offers far more capacity (100 Ah) than either the RB36V40 (40 Ah) or the Vatrer 50 Ah. However, it is heavier (60 lb) and more expensive (≈US $2.2 k) than either. The long warranty and high capacity may be valuable for extended runtime applications but unnecessary if only ~8 Ah is required. |

*Prices are approximate and reflect typical online retailer pricing as of April 2026. Shipping estimates are based on weight and common carrier rates; many retailers offer free shipping for batteries in this price range.

## Discussion and gaps filled

- Lightest option and price:
The FLEX 24 V 12 Ah battery is by far the lightest candidate at ≈4 lb[1]. It costs roughly US $280–340 depending on retailer and offers a 5‑year warranty when registered[2]. The user’s table lists RELiON’s RB24V52, which weighs 29.1 lb and costs about US $930—substantially heavier and more expensive but with greater energy storage and a 10‑year warranty. If low weight and cost are priorities, the FLEX battery is an attractive choice; if long runtime and warranty are more important, the RB24V52 remains compelling.

- Ordering by weight:
When sorted by weight, the batteries considered here rank as follows (lightest to heaviest): FLEX 24 V 12 Ah (≈4 lb)[1] → Vatrer 36 V 50 Ah (≈22 lb)[4] → RELiON InSight 48 V‑LT 30 Ah (≈34.4 lb)[9] → Impulse Lithium 36 V 100 Ah (≈60 lb)[13]. This ordering shows how energy storage and BMS capability increase with weight and price.

- Gaps and differences relative to the user‑provided table:

- Weight and price disparities: The user’s table lists RELiON and AGM options that are significantly heavier and more expensive than the smaller packs above. For instance, the RB48V100 weighs 136.7 lb and costs US $3,495.95, whereas the InSight 48 V‑LT is 34.4 lb and US $1,249.95[11][9].

- Warranty and cycle life: The FLEX battery’s warranty (5 yr) is shorter than the 10‑yr warranties of RELiON and Impulse Lithium packs, but still longer than many consumer tool batteries. Vatrer offers a 5‑yr warranty and claims 6,000+ cycles[16]—more cycles than RELiON’s 3,500‑cycle rating but a shorter warranty.

- Usable energy vs requirement: All lithium packs listed here exceed the user’s minimum ≈288 Wh of nameplate energy (230.65 Wh usable at 80 % depth of discharge). The FLEX battery’s 288 Wh is just at the threshold, while the others offer 1.5–2 kWh or more. Even though the RELiON RB24V52 in the user’s table has far greater capacity (1.28 kWh), the smaller batteries still meet the requirement.

- Peak‑current capability: Each battery’s BMS supports peak currents well above the 2.02 kW requirement: the FLEX battery’s 2.52 kW rating corresponds to ~105 A at 24 V[1]; the Vatrer 36 V pack allows 100 A peak[4]; the RELiON InSight 48 V‑LT supports 300 A peak for 10 s[7]; and the Impulse Lithium battery allows 100 A peak[17]. All therefore satisfy the user’s 2‑kW/peak‑current requirement.

- Lead‑acid options excluded: The user’s table includes heavy lead‑acid (ODYSSEY) strings. Because the request asked only for batteries up to 48 V and emphasised lighter solutions, these lead‑acid options (often >150 lb for a 24 V string) were not revisited here.

In summary, if portability and cost are paramount, the FLEX 24 V 12 Ah and Vatrer 36 V 50 Ah packs offer the best weight‑to‑performance trade‑off. For larger systems needing more energy, the RELiON InSight 48 V‑LT provides a good compromise between weight, capacity and warranty. The Impulse Lithium 36 V 100 Ah battery delivers the highest capacity but is the heaviest and most expensive of the sub‑48‑V options.

[1] [2] 24V 12.0Ah LITHIUM-ION BATTERY by FLEX

https://www.flexpowertools.com/24v-12ah-lithium-ion-battery-fx0231-1/

[3] [4] [5] [6] [16] Vatrer 36V 50Ah LiFePO4 Trolling Motor Battery | 50A BMS + Low-Temp 1920Wh Minn Kota MotorGuide

https://www.lithiumion-batteries.com/products/product/vatrer-36v-50ah-lifepo4-marine-trolling-motor-lithium-battery-with-50a-bms-low-temp-protection-%E2%80%93-lightweight-deep-cycle-power-for-minn-kota-motorguide-all-36v-trolling-motors

[7] [8] [9] [10] [11] [12] RELiON InSight 48V-LT 30Ah GC2/GC8 Cold Weather LiFePO4 Battery

https://www.lithiumion-batteries.com/products/product/insight-series%C2%AE-48v-lt-48v-30ah-cold-weather-deep-cycle-lithium-battery-group-size-gc2-gc8

[13] [15] [17] Impulse Lithium 36v-100Ah Platinum Series LiFePO4 Lithium Battery - Impulse Lithium

https://impulselithium.com/product/impulse-lithium-36v-100ah-platinum-series-lifepo4-lithium-battery/

[14] terms of sale and warranty - Impulse Lithium

https://impulselithium.com/terms-of-sale-warranty/