# Purchasable battery candidates that satisfy the user’s requirements across 24V, 36V, 48V, 60V, and 72V

## Executive summary

Using the user’s inputs (energy need 230.65 Wh; lithium sized with 80% usable and lead‑acid sized with 50% usable) and the user-provided derived minimum peak C‑rates (7.02C lithium, 4.39C lead‑acid), the implied peak power requirement is about 2.02 kW (≈2024 W). This translates to required peak current of roughly 84 A @ 24V, 56 A @ 36V, 42 A @ 48V, 34 A @ 60V, and 28 A @ 72V (current scales inversely with voltage by I=P/V).

The strongest “drop‑in / spec‑sheeted” lithium matches with clear BMS limits were found in LiFePO4 products from RELiON[1] for the 24V/36V/48V classes, while higher-voltage (60V/72V) options are better served by e‑mobility Li‑ion packs (triangle/rectangle formats) from eBikeling[2] and EM3ev[3] with explicit discharge/BMS details. [4]

For lead‑acid, a widely purchasable, extremely high‑pulse option is the Group‑31 AGM ODYSSEY Battery[5] ODX‑AGM31, which can be series‑stacked to achieve 24/36/48/60/72V nominal strings. This meets the user’s power/current requirements with ample margin on paper (very high 5‑second pulse ratings), but at major tradeoffs in mass and energy efficiency typical of lead‑acid. [6]

## Derived requirements from the user table

### Inputs used (from the user prompt)

- Energy requirement: 230.65 Wh

- Sizing assumptions:

- Lithium: 80% usable → minimum nameplate energy =230.65/0.8=288.31 Wh

- Lead‑acid: 50% usable → minimum nameplate energy =230.65/0.5=461.30 Wh

- Derived minimum peak C‑rates (given by user):

- Lithium: 7.02C

- Lead‑acid: 4.39C

### Derived peak power and required peak current by voltage class

Because C_peak=I_peakAh and I_peak=P_peakV while Ah_min=WhV⋅%usable, the voltage cancels and the user’s “min peak C‑rate” implies a single peak power requirement:

- From lithium: P_peak≈7.02⋅230.65/0.8≈2024 W

- From lead‑acid: using 4.39C gives ≈2025 W (difference is rounding in the provided C‑rate)

Using 2024 W as the common peak power requirement, the derived minimum capacities and peak currents are:

| Nominal system voltage | Lithium min capacity @ 80% usable (Ah) | Lead‑acid min capacity @ 50% usable (Ah) | Peak current required for ~2024 W (A) |
| --- | --- | --- | --- |
| 24 V | 12.01 | 19.22 | 84.33 |
| 36 V | 8.01 | 12.81 | 56.22 |
| 48 V | 6.01 | 9.61 | 42.17 |
| 60 V | 4.81 | 7.69 | 33.73 |
| 72 V | 4.00 | 6.41 | 28.11 |

Interpretation: if you buy a lithium pack sized exactly to the energy minimum at each voltage, it must tolerate about 7.02C peak; choosing a larger‑Ah pack reduces the required C‑rate proportionally (while the peak current requirement is still driven by P/V). (All calculations in this section are derived from the user-provided numbers.)

## Candidate batteries that satisfy the requirements

The candidates below are organized by voltage class. For each class, the lithium option is prioritized; a lead‑acid candidate is included as a series string of identical 12V AGM batteries (a common procurement approach for lead‑acid at higher voltages).

### 24V class candidate set

Lithium (LiFePO4) candidate: RELiON RB24V52 (25.6V nominal, 52Ah, Group 31)
Vendor/manufacturer: RELiON (direct product page). Price shown: $929.95. [7]
Datasheet: linked on the RELiON product page (“Download PDF”). [8]

Key specifications (manufacturer stated):
- Nominal voltage: 25.6 V; nominal capacity: 52 Ah; energy: 1280 Wh [8]
- Stated discharge limits: 50 A continuous, 100 A peak (7.5 s ±2.5 s); discharge over‑current protection 215 A ±15 A (10 ±5 ms) [8]
- Chemistry: LiFePO4 (cylindrical cells) [8]
- Form factor: BCI Group 31; dimensions 13.0 × 6.8 × 8.4 in; weight 29.1 lb [8]
- Protection: built‑in BMS; IP67 enclosure rating stated [8]
- Warranty / cycle life: product page states 10‑year warranty and that RELiON batteries “still provide 80% of rated capacity after 3,500 cycles.” [9]
- Series capability: “Maximum Batteries Connected in Series: 1 (Single Use)” (i.e., not intended for series stacking). [8]

Usable capacity against the user requirement:
- Using the user’s lithium assumption (80% usable): 41.6 Ah usable and 1024 Wh usable (0.8×52Ah and 0.8×1280Wh).
- Manufacturer marketing statement says lithium provides “100% of their rated capacity” (if you choose to credit 100% usable, usable energy would be the full 1280 Wh). [8]

Power/current check (derived from stated limits):
- Required peak current at this pack’s nominal voltage: 2024/25.6≈79.1 A.
- Pack capability: 100 A peak → peak power ≈ 2.56 kW at 25.6V; continuous ≈ 1.28 kW at 50A. [8]

Justification: This pack exceeds the 230.65 Wh usable‑energy requirement by a wide margin even under the conservative 80% usable rule, and its 100A peak discharge rating covers the ~79A peak current implied by the user’s ~2.02 kW peak power requirement. [8]

Lead‑acid (AGM) candidate: ODYSSEY ODX‑AGM31 (12V, 103Ah) ×2 in series → 24V nominal string
Vendor/manufacturer: ODYSSEY online store shows $588.99 per battery. [10]
Warranty document: ODYSSEY/NORTHSTAR general limited warranty (North America) lists ODYSSEY Extreme (ODX prefix) … 4 years for engine starting for BCI/DIN sizes (application‑dependent). [11]

Per‑battery specs (manufacturer stated on product page):
- Voltage: 12V; 20‑hr capacity: 103Ah [10]
- Peak current metric: PHCA 2150 A (5 seconds) [10]
- Dimensions: 13 × 6.8 × 9.5 in (terminals included); weight 77.8 lb [10]
- Cycle life claim: product page states “up to 400 cycles at 80% depth of discharge.” [10]

24V string (2 identical units in series) “system‑level” values:
- Nominal voltage: 24V (series)
- Capacity: 103Ah nameplate (series keeps Ah constant)
- Usable capacity under the user’s lead‑acid assumption (50% usable): 51.5Ah usable, 1236Wh usable
- Pulse capability: still limited by the series string current, with the same 2150A (5s) PHCA metric (real‑world voltage sag will constrain usable power). [10]

Justification: Even at 50% usable, the 24V AGM string far exceeds 230.65Wh usable energy, and its published 5‑second PHCA pulse current is orders of magnitude above the ~84A peak current implied by the user’s peak power (noting that lead‑acid voltage sag and heating often limit sustained high‑power draw). [10]

### 36V class candidate set

Lithium (LiFePO4) candidate: RELiON RB36V40 (38.4V nominal, 40Ah, Group 31)
Vendor: RELiON product page lists $1,089.95 and points to online purchase; Del City[12] lists the same item as in stock and shows list price and a sale price. [13]
Datasheet (PDF): RB36V40 data sheet. [14]

Key specifications (manufacturer stated):
- Nominal voltage: 38.4V; capacity: 40Ah; energy: 1536Wh [15]
- Discharge limits: 50A continuous, 175A peak (7.5s ±2.5s); over‑current protection shown as 200A ±35A (timing specified). [15]
- Chemistry: LiFePO4; enclosure rating shown as IP67 on datasheet [14]
- Form factor: Group 31; dimensions 13 × 6.8 × 8.4 in; weight 30.9 lb [15]
- Warranty: product page states 10‑year warranty. [16]
- Cycle life (brand-level): RELiON’s catalog states LiFePO4 is “typically rated to deliver 3,500 cycles at 80% DOD,” and also asserts “80% of rated capacity after 3,500 cycles” (general RELiON LiFePO4 positioning). [17]

Usable capacity against the user requirement:
- At 80% usable: 32Ah usable, 1229Wh usable (0.8×1536Wh).

Power/current check:
- Required peak current at 38.4V: 2024/38.4≈52.7 A.
- Battery ratings: 175A peak is well above required peak current; 50A continuous is slightly below the 52.7A implied by a fully sustained 2.02kW load at 38.4V. [15]

Justification: The RB36V40 comfortably covers the required ~2.02kW peak via its 175A peak rating and greatly exceeds the energy requirement at 80% usable; if the application’s 2kW demand is sustained longer than the ~7.5s peak window, paralleling packs (supported on the datasheet) is the more conservative approach to raise continuous current headroom. [15]

Lead‑acid (AGM) candidate: ODYSSEY ODX‑AGM31 ×3 in series → 36V nominal string
The same per‑battery specs apply as above. [6]

36V string values:
- Usable energy at 50% usable: 1854Wh usable
- Published pulse current metric: PHCA 2150A (5s) per string (current common through series). [10]

Justification: Meets energy requirement with wide margin and has ample published short-pulse current capability versus the ~56A peak current requirement for 36V class, with the same practical caveat that lead‑acid voltage sag makes sustained high‑power operation less efficient. [10]

### 48V class candidate set

Lithium (LiFePO4) candidate: RELiON RB48V100 (51.2V nominal, 100Ah)
Vendor: RELiON product page shows $3,495.95 and 10‑year warranty; Del City lists a sale price and in stock status. [18]
Datasheet: linked from the product page (“Download PDF”). [19]

Key specifications (manufacturer stated):
- Nominal voltage: 51.2V; capacity: 100Ah; energy: 5120Wh [20]
- Discharge limits: 200A continuous, 250A peak (4s ±1s); over‑current protection 300A (5s) [20]
- Chemistry: LiFePO4; enclosure: IP65; steel case [20]
- Dimensions: 32.8 × 11.2 × 10.0 in; weight 136.7 lb [20]
- Warranty / cycle life claims: 10‑year warranty and “80% of rated capacity after 3,500 cycles” appear on the product page. [21]

Usable capacity (user assumption):
- At 80% usable: 80Ah usable, 4096Wh usable.

Power/current check:
- Required peak current at 51.2V: 2024/51.2≈39.5 A.
- Battery supports 200A continuous (≈10.24kW continuous at 51.2V). [22]

Justification: Easily satisfies both the energy minimum and the ~2kW peak power requirement, with continuous current capability far exceeding the ~40A implied by 2.02kW at 51.2V; the limiting factor is unlikely to be discharge current and more likely mechanical/space/weight. [20]

Lead‑acid (AGM) candidate: ODYSSEY ODX‑AGM31 ×4 in series → 48V nominal string
Per-battery specs and warranty architecture as above. [6]

Justification: The 48V AGM string exceeds the 230.65Wh usable requirement at 50% usable and has published short‑pulse current far above the ~42A peak current implied at 48V class, but is physically very large and heavy (four Group‑31 AGM units). [10]

### 60V class candidate set

Lithium (Li‑ion) candidate: eBikeling “60V 20Ah Triangle Battery” (62V nominal, 20Ah)
Vendor: eBikeling product page lists $430.00. [23]
Warranty: eBikeling warranty page states 1‑year warranty on new hub motors and batteries. [24]

Key specifications (vendor stated):
- Nominal voltage: 62V; nominal capacity: 20Ah; triangle form factor [23]
- Discharge: “Rated discharge current 60A,” and “Max discharge current 60A,” with “Overcurrent protection 65A” [23]
- Cycle life (vendor stated): 1000 cycles [23]
- Cell model: INR21700M50LT (a lithium‑ion cylindrical cell family commonly supplied by LG Energy Solution[25]; publicly available characterization of the LG M50 family reports an NMC‑811 cathode formulation). [26]
- Dimensions: 14 × 11 × 7.6 in; weight 18.0 lb [23]

Usable capacity (user assumption):
- At 80% usable: 16Ah usable, ≈992Wh usable.

Power/current check:
- Required peak current at 62V: 2024/62≈32.6 A.
- Pack allows up to 60A discharge, implying ≈ 3.72kW at 62V. [23]

Justification: Meets the required usable energy with substantial margin (even at 80% usable) and supports the ~33–34A peak current implied by a ~2kW load in the 60V class, with explicit over‑current protection indicating a BMS‑enforced discharge ceiling. [27]

Lead‑acid (AGM) candidate: ODYSSEY ODX‑AGM31 ×5 in series → 60V nominal string
Per-battery specs and warranty architecture as above. [6]

Justification: Meets energy requirement at 50% usable and has extremely large short‑pulse current headroom relative to the ~34A peak current implied at 60V class; major downside is mass/volume of five Group‑31 AGM batteries in series. [10]

### 72V class candidate set

Lithium (Li‑ion) candidate: EM3ev 72V eBike Battery 15Ah (1080Wh), Samsung 21700 pack, Bluetooth Smart BMS
Vendor: EM3ev product page shows $449.00 (price varies by configuration) and indicates In Stock. [28]
Warranty: EM3ev warranty policy states battery products come with a 1‑year warranty and notes many issues can be resolved by replacing protection board / BMS-related components. [29]

Key specifications (vendor stated):
- Nominal voltage: 72V; full charge about 84V; capacity 15Ah; energy 1080Wh [28]
- Discharge: 75A max continuous discharge [28]
- BMS: “Smart BMS + cell balancing + Bluetooth” [28]
- Dimensions: 158 × 81 × 298 mm; weight 6.5 kg [30]
- Cycle life (brand-level): EM3ev’s battery catalog page states “Long cycle life: 800+ cycles while retaining most of the original capacity when operated correctly.” [31]

Usable capacity (user assumption):
- At 80% usable: 12Ah usable, 864Wh usable.

Power/current check:
- Required peak current at 72V: 2024/72≈28.1 A.
- Pack supports 75A continuous, i.e., ≈ 5.4kW at 72V nominal. [28]

Justification: Easily satisfies the energy minimum and supports the required peak power/current with large headroom (75A continuous vs ~28A required), while also providing explicit smart‑BMS functionality and a documented 1‑year warranty process for BMS/protection-board repairs. [32]

Lead‑acid (AGM) candidate: ODYSSEY ODX‑AGM31 ×6 in series → 72V nominal string
Per-battery specs and warranty architecture as above. [6]

Justification: Meets usable energy requirement at 50% usable and has extremely high published pulse capability relative to the ~28A peak current implied at 72V class, but is typically impractical in weight/space for most mobile 72V applications. [10]

## Cross-voltage comparison table

Assumptions used for “usable” column: lithium uses 80% usable (per user constraint) unless a different value is explicitly stated by the vendor; lead‑acid uses 50% usable (per user constraint). Where peak duration is stated, it is shown.

| Voltage class | Candidate | Chemistry | Nominal V | Usable capacity (Ah / Wh) | Discharge rating (continuous / peak) | Max discharge power (continuous / peak) | Weight | Price | Warranty | Cycle life |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24V | RELiON RB24V52 | LiFePO4 | 25.6V [8] | 41.6Ah / 1024Wh (80% assumed); vendor claims 100% usable [8] | 50A / 100A (7.5s) [8] | 1.28kW / 2.56kW [8] | 29.1 lb [8] | $929.95 [9] | 10 years [9] | 80% capacity after 3,500 cycles (brand claim on page) [8] |
| 24V | ODYSSEY ODX‑AGM31 ×2 series | AGM lead‑acid | 24V (2×12V) [10] | 51.5Ah / 1236Wh (50% usable) | Not stated as “max continuous”; PHCA 2150A (5s) pulse spec [10] | (pulse spec) 51.6kW theoretical @24V | 155.6 lb | $1177.98 | Up to 4‑year (ODX, BCI sizes, engine start) [11] | “Up to 400 cycles @ 80% DOD” claim on product page [10] |
| 36V | RELiON RB36V40 | LiFePO4 | 38.4V [15] | 32Ah / 1229Wh (80% assumed) | 50A / 175A (7.5s) [15] | 1.92kW / 6.72kW [15] | 30.9 lb [15] | $1,089.95 list; Del City shows in‑stock (price varies) [13] | 10 years [16] | 3,500 cycles @ 80% DOD (RELiON catalog statement) [17] |
| 36V | ODYSSEY ODX‑AGM31 ×3 series | AGM lead‑acid | 36V (3×12V) [10] | 51.5Ah / 1854Wh (50% usable) | PHCA 2150A (5s) pulse spec [10] | (pulse spec) 77.4kW theoretical @36V | 233.4 lb | $1766.97 | Up to 4‑year (application‑dependent) [11] | Up to 400 cycles @ 80% DOD claim [10] |
| 48V | RELiON RB48V100 | LiFePO4 | 51.2V [22] | 80Ah / 4096Wh (80% assumed); vendor claims 100% usable [19] | 200A / 250A (4s) [22] | 10.24kW / 12.8kW [22] | 136.7 lb [22] | $3,495.95 (sale pricing varies) [33] | 10 years [19] | 80% capacity after 3,500 cycles (page claim) [19] |
| 48V | ODYSSEY ODX‑AGM31 ×4 series | AGM lead‑acid | 48V (4×12V) [10] | 51.5Ah / 2472Wh (50% usable) | PHCA 2150A (5s) pulse spec [10] | (pulse spec) 103.2kW theoretical @48V | 311.2 lb | $2355.96 | Up to 4‑year (application‑dependent) [11] | Up to 400 cycles @ 80% DOD claim [10] |
| 60V | eBikeling 60V 20Ah Triangle | Li‑ion (21700) | 62V [23] | 16Ah / ~992Wh (80% assumed) | 60A / 60A (max listed), over‑current 65A [23] | 3.72kW / 3.72kW [23] | 18.0 lb [23] | $430.00 [23] | 1 year [24] | 1000 cycles (vendor stated) [23] |
| 60V | ODYSSEY ODX‑AGM31 ×5 series | AGM lead‑acid | 60V (5×12V) [10] | 51.5Ah / 3090Wh (50% usable) | PHCA 2150A (5s) pulse spec [10] | (pulse spec) 129kW theoretical @60V | 389.0 lb | $2944.95 | Up to 4‑year (application‑dependent) [11] | Up to 400 cycles @ 80% DOD claim [10] |
| 72V | EM3ev 72V 15Ah Samsung 21700 pack | Li‑ion (21700) | 72V [28] | 12Ah / 864Wh (80% assumed) | 75A continuous [28] | 5.40kW continuous [28] | 6.5 kg (≈14.3 lb) [30] | $449.00 [28] | 1 year [29] | 800+ cycles (vendor family claim) [31] |
| 72V | ODYSSEY ODX‑AGM31 ×6 series | AGM lead‑acid | 72V (6×12V) [10] | 51.5Ah / 3708Wh (50% usable) | PHCA 2150A (5s) pulse spec [10] | (pulse spec) 154.8kW theoretical @72V | 466.8 lb | $3533.94 | Up to 4‑year (application‑dependent) [11] | Up to 400 cycles @ 80% DOD claim [10] |

## Selection logic and procurement notes

### Flowchart of selection criteria

flowchart TD
  A[Start: choose nominal voltage class\n24 / 36 / 48 / 60 / 72] --> B[Compute minima from user table\nUsable Wh >= 230.65\nPeak power ~2024 W\nPeak current = 2024 / V]
  B --> C{Chemistry preference?}
  C -->|Lithium preferred| D[Filter lithium packs\nUsable energy check\nBMS discharge limits >= I_peak\nVerify peak-duration window]
  C -->|Lead-acid acceptable| E[Build series string of 12V units\nUsable energy check at 50% usable\nCheck published pulse current\nAssess mass + sag risk]
  D --> F[Confirm mechanical fit\nForm factor: Group 31 / triangle / rectangle\nDimensions + weight]
  E --> F
  F --> G[Procurement check\nIn-stock / shipping constraints\nWarranty + cycle life]
  G --> H[Select candidate(s)\nOptionally: parallel packs to raise continuous A]

### Lead time / availability signals explicitly provided by vendors

- eBikeling states orders are processed “within 1 business day” and shows an estimated delivery window of “2 to 5 business days” on the product page. [23]

- EM3ev labels the 72V 15Ah pack listing as “In Stock.” [28]

- Del City explicitly marks the RB36V40 listing as “✓ IN STOCK” (pricing can vary vs. MSRP). [34]

If you want a timeline view for the above signals:

timeline
  title Procurement signals (vendor-stated)
  Day 0 : Order placed
  Day 0-1 : eBikeling processes within 1 business day
  Day 2-5 : eBikeling estimated delivery 2–5 business days
  Day 0 : EM3ev listing shows "In Stock"
  Day 0 : Del City listing shows "In Stock" (RB36V40)

### Important engineering caveats tied to the user’s constraints

- Peak vs continuous: Several lithium packs list distinct peak windows (e.g., 7.5s or 4s). If your “peak” event is longer (or repeated with short rest), treat continuous current and thermal limits as the binding constraint; this is the main reason the RB36V40 is “peak‑safe” but only borderline for a sustained 2.02kW draw at 38.4V. [35]

- Lead‑acid pulse specs don’t equal sustained power: AGM products like ODX‑AGM31 publish very high short‑pulse metrics (PHCA), which will exceed your ~2kW requirement on paper, but sustained 2kW discharge is typically constrained by voltage sag, heating, and efficiency losses inherent to lead‑acid under high‑rate discharge (the user’s 50% usable assumption already reflects typical conservative usage). [10]

- Chemistry differences matter for usable fraction: RELiON explicitly markets “100% usable capacity” for lithium vs typical 50% for lead‑acid; your sizing rule uses 80% for lithium, so all lithium candidates here exceed the energy minimum with large margin even under your conservative rule. [36]

- High‑voltage compatibility: EM3ev notes a “72V” pack charges to about 84V, so motor controller over‑voltage limits and LVC settings must be compatible (a wiring/controls constraint outside the battery’s Wh/A ratings). [28]

[1] [28] [31] [32] https://em3ev.com/ebike-batteries/

https://em3ev.com/ebike-batteries/

[2] [24] Warranty, Returns & Replacements

https://ebikeling.com/pages/warranty-ebikeling-ebike-conversion-kit?srsltid=AfmBOooDLJ6KlnLQMtHzQvnu4dq0snYxB3o1l0gPnuZ9glGpjKAQ06Ry&utm_source=chatgpt.com

[3] [12] [13] [15] [16] [35] https://relionbattery.com/products/lithium/rb36v40

https://relionbattery.com/products/lithium/rb36v40

[4] [7] [8] [9] [36] https://relionbattery.com/products/lithium/rb24v52?srsltid=AfmBOopas5XlLkddmPR7xq-E34-aSogDhWJKBV_XWnS8HURduNL4o_Ee

https://relionbattery.com/products/lithium/rb24v52?srsltid=AfmBOopas5XlLkddmPR7xq-E34-aSogDhWJKBV_XWnS8HURduNL4o_Ee

[5] [6] [10] [25] https://www.odysseybattery.com/products/odx-agm31-battery-31-pc2150s/

https://www.odysseybattery.com/products/odx-agm31-battery-31-pc2150s/

[11] https://www.odysseybattery.com/wp-content/uploads/sites/2/odyssey-battery-general-product-limited-warranty.pdf

https://www.odysseybattery.com/wp-content/uploads/sites/2/odyssey-battery-general-product-limited-warranty.pdf

[14] https://ceb8596f236225acd007-8e95328c173a04ed694af83ee4e24c15.ssl.cf5.rackcdn.com/docs/product/RELiON-Data-Sheet_RB36V40.pdf

https://ceb8596f236225acd007-8e95328c173a04ed694af83ee4e24c15.ssl.cf5.rackcdn.com/docs/product/RELiON-Data-Sheet_RB36V40.pdf

[17] https://ceb8596f236225acd007-8e95328c173a04ed694af83ee4e24c15.ssl.cf5.rackcdn.com/docs/product/REL-024-CAT-0423-RELiON20202420Catalogue-compressed.pdf

https://ceb8596f236225acd007-8e95328c173a04ed694af83ee4e24c15.ssl.cf5.rackcdn.com/docs/product/REL-024-CAT-0423-RELiON20202420Catalogue-compressed.pdf

[18] [19] [20] [21] [22] [33] https://relionbattery.com/products/lithium/rb48v100?srsltid=AfmBOooyxhh2_Rr5Rk3k3SfXnIwT10cCzSgsSo8i9ZTAzXHz2W6oBAXz

https://relionbattery.com/products/lithium/rb48v100?srsltid=AfmBOooyxhh2_Rr5Rk3k3SfXnIwT10cCzSgsSo8i9ZTAzXHz2W6oBAXz

[23] [26] [27]  60V 20AH Triangle Electric Bike Battery - Waterproof – eBikeling

https://ebikeling.com/products/60v-20ah-triangle-ebike-battery?srsltid=AfmBOoqh4s9K7SioYXuvxQ30EHU3eS5sKg3WLGSQ8aPp0zLjYtzyj5Rh

[29] https://em3ev.com/warranty-after-sales-policy/

https://em3ev.com/warranty-after-sales-policy/

[30] 72V eBike Battery | 15Ah 1080Wh 75A Samsung 21700 Pack | EM3ev

https://em3ev.com/ebike-batteries/72v/

[34] https://www.delcity.net/store/battery-cables-accessories/batteries/relion-lithium/legacy-series-36v/RB36V40/?srsltid=AfmBOooIWfpUSqJKhxVuZYfZfI8b3UXo10NWgcq4iGqQRKE3aHfEvJjV

https://www.delcity.net/store/battery-cables-accessories/batteries/relion-lithium/legacy-series-36v/RB36V40/?srsltid=AfmBOooIWfpUSqJKhxVuZYfZfI8b3UXo10NWgcq4iGqQRKE3aHfEvJjV